RULE 306.	DUST AND FUMES - NORTHERN ZONE.  (Adopted 10/18/1971, readopted 10/23/1978)

A.	A person shall not discharge, in any one hour, from any source, dust or fumes in total quantities in excess of the amount shown in Table 306(a).

B.	To use this table, determine the "Process Weight Per Hour."  Then find this figure on the table, opposite which is the maximum number of pounds of contaminants which may be discharged into the atmosphere in any one hour.  Where the Process Weight Per Hour falls between figures in the left-hand column, the exact weight of permitted discharge may be interpolated.

Table 306(a)

Process Weight Rate

Northern Zone

| Process Weight/ Hour (lbs) | Maximum Weight Discharge/ Hour (lbs) | Process Weight/ Hour (lbs) | Maximum Weight Discharge/ Hour (lbs) |
| --- | --- | --- | --- |

| 50 | 0.24 | 3300 | 5.36 |
| --- | --- | --- | --- |
| 100 | 0.46 | 3400 | 5.44 |
| 150 | 0.66 | 3500 | 5.52 |
| 200 | 0.85 | 3600 | 5.61 |
| 300 | 1.20 | 3700 | 5.69 |
| 350 | 1.35 | 3800 | 5.77 |
| 400 | 1.50 | 3900 | 5.85 |
| 450 | 1.63 | 4000 | 5.93 |
| 500 | 1.77 | 4100 | 6.01 |
| 550 | 1.89 | 4200 | 6.08 |
| 600 | 2.01 | 4300 | 6.15 |
| 650 | 2.12 | 4400 | 6.22 |
| 700 | 2.24 | 4500 | 6.30 |
| 750 | 2.34 | 4600 | 6.37 |
| 800 | 2.43 | 4700 | 6.45 |
| 850 | 2.53 | 4800 | 6.52 |
| 900 | 2.62 | 4900 | 6.60 |
| 950 | 2.72 | 5000 | 6.67 |
| 1000 | 2.80 | 5500 | 7.03 |
| 1100 | 2.97 | 6000 | 7.37 |
| 1200 | 3.12 | 6500 | 7.71 |
| 1300 | 3.26 | 7000 | 8.05 |
| 1400 | 3.40 | 7500 | 8.39 |
| 1500 | 3.54 | 8000 | 8.71 |
| 1600 | 3.66 | 8500 | 9.03 |
| 1700 | 3.79 | 9000 | 9.36 |
| 1800 | 3.91 | 9500 | 9.67 |
| 1900 | 4.03 | 10000 | 10.00 |
| 2000 | 4.14 | 11000 | 10.63 |
| 2100 | 4.24 | 12000 | 11.28 |
| 2200 | 4.34 | 13000 | 11.89 |
| 2300 | 4.44 | 14000 | 12.50 |
| 2400 | 4.55 | 15000 | 13.13 |
| 2500 | 4.64 | 16000 | 13.74 |
| 2600 | 4.74 | 17000 | 14.36 |
| 2700 | 4.84 | 18000 | 14.97 |
| 2800 | 4.92 | 19000 | 15.58 |
| 2900 | 5.02 | 20000 | 16.19 |
| 3000 | 5.10 | 30000 | 22.22 |
| 3100 | 5.18 | 40000 | 28.30 |
| 3200 | 5.27 | 50000 | 34.30 |
|  |  | 60000 or more | 40.00 |