RULE 305.	PARTICULATE MATTER CONCENTRATION - SOUTHERN ZONE.  (Adopted 5/1/1972, readopted 10/23/1978)

A person shall not discharge into the atmosphere from any source, particulate matter in excess of the concentration shown in Table 305(a).

Where the volume discharged falls between figures listed in the table, the exact concentration permitted to be discharged shall be determined by linear interpolation.

The provisions of this Rule shall not apply to emissions resulting from the combustion of liquid or gaseous fuels in steam generators or gas turbines.

Table 305(a)

Particulate Matter Concentration

Southern Zone

| Volume Discharged - Cubic Feet per Minute Calculated as Dry Gas at Standard Conditions | Maximum Concentration of Particulate Matter Allowed in Discharged Gas - Grains per Cubic Foot of Dry Gas at Standard Conditions | Volume Discharged - Cubic Feet per Minute Calculated as Dry Gas at Standard Conditions | Maximum Concentration of Particulate Matter Allowed in Discharged Gas - Grains per Cubic Foot of Dry Gas at Standard Conditions |
| --- | --- | --- | --- |
| 1000 or less | 0.200 | 20000 | 0.0635 |
| 1200 | 0.187 | 30000 | 0.0544 |
| 1400 | 0.176 | 40000 | 0.0487 |
| 1600 | 0.167 | 50000 | 0.0447 |

| 1800 | 0.160 | 60000 | 0.0417 |
| --- | --- | --- | --- |
| 2000 | 0.153 | 70000 | 0.0393 |
| 2500 | 0.141 | 80000 | 0.0374 |
| 3000 | 0.131 | 100000 | 0.0343 |

| 3500 | 0.124 | 200000 | 0.0263 |
| --- | --- | --- | --- |
| 4000 | 0.118 | 400000 | 0.0202 |
| 5000 | 0.108 | 600000 | 0.0173 |
| 6000 | 0.101 | 800000 | 0.0155 |

| 7000 | 0.0949 | 1000000 | 0.0142 |
| --- | --- | --- | --- |
| 8000 | 0.0902 | 1500000 | 0.0122 |
| 10000 | 0.0828 | 2000000 | 0.0109 |
| 15000 | 0.0709 | 2500000 or more | 0.0100 |