RULE 307.	PARTICULATE MATTER EMISSION WEIGHT RATE - SOUTHERN ZONE.  (Adopted 5/1/1972, readopted 10/23/1978)

A.	"Solid Particulate Matter" includes any material which would become solid particulate matter if cooled to standard conditions.

B.	A person shall not discharge into the atmosphere from any source, solid particulate matter in excess of the rate shown in Table 307(a).

Table 307(a)

Particulate Matter Emission Weight Rate

Southern Zone

| Process Weight Per Hour - Pounds Per Hour | Maximum Discharge Rate Allowed For Solid Particulate Matter (Aggregate Discharged From All Points of Process) - Pounds Per Hour | Process Weight Per Hour - Pounds Per Hour | Maximum Discharge Rate Allowed For Solid Particulate Matter (Aggregate Discharged From All Points of Process)- Pounds Per Hour |
| --- | --- | --- | --- |

| 250 or less | 1.00 | 12000 | 10.4 |
| --- | --- | --- | --- |
| 300 | 1.12 | 14000 | 10.8 |
| 350 | 1.23 | 16000 | 11.2 |
| 400 | 1.34 | 18000 | 11.5 |
| 450 | 1.44 | 20000 | 11.8 |
| 500 | 1.54 | 25000 | 12.4 |
| 600 | 1.73 | 30000 | 13.0 |
| 700 | 1.90 | 35000 | 13.5 |
| 800 | 2.07 | 40000 | 13.9 |
| 900 | 2.22 | 45000 | 14.3 |
| 1000 | 2.38 | 50000 | 14.7 |
| 1200 | 2.66 | 60000 | 15.3 |
| 1400 | 2.93 | 70000 | 15.9 |
| 1600 | 3.19 | 80000 | 16.4 |
| 1800 | 3.43 | 90000 | 16.9 |
| 2000 | 3.66 | 100000 | 17.3 |
| 2500 | 4.21 | 120000 | 18.1 |
| 3000 | 4.72 | 140000 | 18.8 |
| 3500 | 5.19 | 160000 | 19.4 |
| 4000 | 5.64 | 180000 | 19.9 |
| 4500 | 6.07 | 200000 | 20.4 |
| 5000 | 6.49 | 250000 | 21.6 |
| 5500 | 6.89 | 300000 | 22.5 |
| 6000 | 7.27 | 350000 | 23.4 |
| 6500 | 7.64 | 400000 | 24.1 |
| 7000 | 8.00 | 450000 | 24.8 |
| 7500 | 8.36 | 500000 | 25.4 |
| 8000 | 8.70 | 600000 | 26.6 |
| 8500 | 9.04 | 700000 | 27.6 |
| 9000 | 9.36 | 800000 | 28.4 |
| 9500 | 9.68 | 900000 | 29.3 |
| 10000 | 10.00 | 1000000 or more | 30.0 |